MECE308 Project Description

2025-2026 (II)

# Sightseeing Pedal Pub Bike with Beverage Bar

## Project Description

This project involves the design and development of a Sightseeing Pedal Pub Bike equipped with a beverage-serving bar. The vehicle will be primarily human-powered during sightseeing tours, with an auxiliary electric drive system used only for positioning and low-speed maneuvering. The design must prioritize low total weight, minimum electric power consumption, and overall cost efficiency, while maintaining structural safety and user comfort.

## General Specifications

- Capacity

  - 6 pedalers (total estimated weight: 450 kg)

  - 1 driver (estimated weight: 75 kg)

- Operation Modes

  - Sightseeing mode: Vehicle is powered solely by pedaling. No electric power will be engaged during normal operation.

  - Positioning/transport mode: The driver can operate the vehicle using the electric motor to reach the starting point or for operational adjustments.

- Speed Limit

  - Maximum speed in both pedaling and electric-assisted modes: 30 km/h.

  - Sightseeing, whether in pedaling or electric-assisted mode, will take place on zero-slope paths.

- Drive System

  - An electric motor powered by a rechargeable battery.

  - A custom-designed gearbox will reduce motor speed to the required wheel speed.
(Commercial motor–gearbox assemblies must not be used.)

  - The battery capacity allows 2 hours of operation under electric drive.

  - Motor speed—and correspondingly vehicle speed—will be controlled by regulating motor RPM.
No mechanical gear shifting.

- Chassis & Body

  - Four-wheel configuration for stability.

  - Frame/chassis constructed from box-profile steel beams or aluminum box profiles.

  - Seat pedals will incorporate an idle/freewheel mechanism to allow coasting.

  - A lightweight overhead shade canopy will be installed to improve passenger comfort.

- Additional Mechanical Considerations (added for completeness)

  - Steering system designed for safe low-speed maneuverability (likely front-wheel steer).

  - Braking system suitable for full-load conditions, such as hydraulic or mechanical disc brakes.

  - A chain-drive system linking each pedal station to the main drive shaft.

  - Vibration and noise-damping elements where applicable.

  - Beverage bar mounted centrally with secure cupholders and spill-resistant surfaces.

## Project Report Requirements

The final submitted report must include:

- Conceptual Design

  - Overall layout, aesthetics, user placement, and system functionality.

- Mechanical Design and Analysis

  - Structural design of the frame and supports.

  - Strength calculations for beams, joints, and load-bearing components.

  - Pedal drive system design, shaft analysis, and drivetrain layout.

  - Finite element analysis (if applicable).

- Selection of Commercial Components

  - Battery, electric motor (without gearbox), brake system, steering components.

  - Seats/saddles, pedals, chain, sprockets, clutch mechanism, wheels, lighting, etc.

- Gearbox Design

  - Complete design of a speed-reducing gearbox for the electric motor.

  - Gear ratio calculations, material selection, and performance analysis.

- Connection & Assembly Drawings

  - Detailed technical drawings showing:

    - Part connections

    - Welds, bolts, joints

    - Subsystem integration

  - Fully assembled system drawing.

## Reference Images

Photos of an existing 12-person pedal pub bike (engine-driven model with non-functional pedals) are provided for reference and comparison.

PROJECT EVALUATION POLICY

| CONTENT | Grade point | G1 | G2 | G3 | G4 | G5 | G6 | G7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title Page | 1 |  |  |  |  |  |  |  |
| ABSTRACT | 3 |  |  |  |  |  |  |  |
| Table of Contents | 1 |  |  |  |  |  |  |  |
| List of Figure | 1 |  |  |  |  |  |  |  |
| List of Tables | 1 |  |  |  |  |  |  |  |
| INTRODUCTION | 6 |  |  |  |  |  |  |  |
| Project Definition | 3 |  |  |  |  |  |  |  |
| Projects Objectives | 3 |  |  |  |  |  |  |  |
| LITERATURE REVIEW | 10 |  |  |  |  |  |  |  |
| Project Background | 2 |  |  |  |  |  |  |  |
| Previous Work | 4 |  |  |  |  |  |  |  |
| Comparative Work | 4 |  |  |  |  |  |  |  |
| SYSTEM DESIGN and MANUFACTURING | 55 |  |  |  |  |  |  |  |
| Design Constraints and Design Methodology | 5 |  |  |  |  |  |  |  |
| Engineering Design and Manufacturing Standards | 5 |  |  |  |  |  |  |  |
| Product Subsystems, Design Calculations, Selection of Components | 35 |  |  |  |  |  |  |  |
| Technical drawing of the system assembly (simple) | 10 |  |  |  |  |  |  |  |
| PROJECT MANAGEMENT | 9 |  |  |  |  |  |  |  |
| Project Plan (Gantt chart) | 2 |  |  |  |  |  |  |  |
| Contribution of Team Members | 2 |  |  |  |  |  |  |  |
| Challenges and Decision-Making | 2 |  |  |  |  |  |  |  |
| Project Bill of Materials | 3 |  |  |  |  |  |  |  |
| LIFELONG LEARNING | 8 |  |  |  |  |  |  |  |
| CONCLUSIONS and FUTURE RECOMMENDATIONS | 3 |  |  |  |  |  |  |  |
| REFERENCES | 2 |  |  |  |  |  |  |  |
| Neatness, clarity and English use in project report | 10 |  |  |  |  |  |  |  |
| OVERALL | 110 |  |  |  |  |  |  |  |